PUMP AROUND OPERATION:

Description

The work covered by this section consists of furnishing, installing, maintaining and removing any and all pump around operations used on this project.  The Contractor shall install a pump around operation in locations chosen by the contractor and approved by the Engineer.  The diversion pumping shall provide a passageway for the stream flow around the work site.

The quantity of diversion pumping may be increased, decreased, or eliminated entirely as directed.  Such variations in quantity will not be considered as alterations in the details of construction or a change in the character of the work.  See example pump around operation detail on the plans.

Construction Methods

Install a temporary impervious dike as shown on the detail.  Pump water around the work site.  If the water is turbid or exposed to bare soil, pump through a special stilling basin.  Follow detail for the pump around operation.  Once the work is complete in an area remove the impervious dike and diversion pumping materials and equipment.  Place structures in the area and stabilize immediately following removal of the pump around operation.

Measurement and Payment

Temporary impervious dikes will be considered incidental to the diversion pumping.

The pump around operation will be measured and paid for as lump sum for Diversion Pumping.  This measurement shall include multiple installations and removals of the pump around operation.

The above prices and payments will be full compensation for all work covered by this section including, but not limited to, furnishing all of the necessary materials, construction, maintenance and removal of the impervious dike and pump around operation.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Diversion Pumping | Lump Sum |