ROCK A- VANE:

Description

This work consists of the construction and maintenance of physical barriers placed in and along the stream at locations designated on the plans to direct the stream flow (thalweg) toward the center of the channel and to provide grade control.

The quantity of rock cross vanes to be installed will be affected by the actual conditions that occur during the construction of the project.  The quantity of rock cross vanes may be increased, decreased, or eliminated entirely as directed.  Such variations in quantity will not be considered as alterations in the details of construction or a change in the character of the work.

Materials

Refer to Division 10 of NCDOT 2024 Standard Specifications for Roads and Structures

| Item | Section |
| --- | --- |
| Boulder | 1042 and SP for Structure Stone |
| No. 57 Stone | 1005 |
| Riprap, Class A | 1042 |
| Geotextile for Drainage, Type 2 | 1056 |

Boulders shall be used as header and footer rocks for this device.

Construction Methods

Rock cross vanes shall be constructed in accordance with the Rock A-Vane Detail shown in the plans or as directed.  Two vanes, each approximately 1/3 of the stream channel’s bankfull width, will form a 20– 30 angle out from the streambank toward upstream.  The top elevation of both vanes will decrease from bankfull elevation toward the center of the channel at a slope of 4 to 10 percent.  A vane running perpendicular to the stream’s flow will connect the two outside vanes on the upstream end.  Install header and footer rocks according to the detail and plate the upstream side with Geotextile, Type 2 and No. 57 stone.  Voids between the header and footer rocks can be filled with hand-placed Class A riprap as directed.  Footer rocks shall be placed such that the header rock is at streambed elevation.  The rock a-vane shall be keyed into the bank at the downstream end as shown on the Rock A-Vane Detail.

Measurement and Payment

Boulders will be measured and paid for as provided in the Structure Stone Special Provision.

No. 57 Stone will be measured and paid for as provided in the Structure Stone Special Provision.

Riprap, Class A will be measured and paid for as provided in the Structure Stone Special Provision.

Geotextile for Drainage Type 2 will be measured and paid for as provided in the Structure Stone Special Provision.

Such price and payment will be full compensation for all work covered by this section, including, but not limited to furnishing all materials, labor, equipment, and incidentals necessary to construct the rock cross vanes.