Rock Sill:

Description:

The work covered by this section consists of the construction and maintenance of grade control structures placed in and along the stream at locations designated on the plans to prevent the down cutting of freshly graded channel bed.

The quantity of rock sills to be installed will be affected by the actual conditions that occur during the construction of the project.  The quantity of rock sills may be increased, decreased, or eliminated entirely at the direction of the Engineer.  Such variations in quantity will not be considered as alterations in the details of construction or a change in the character of the work.

Materials

Refer to Division 10 of NCDOT 2024 Standard Specifications for Roads and Structures

| Item | Section |
| --- | --- |
| Boulder | 1042 and SP for Structure Stone |
| No. 57 Stone | 1005 |
| Riprap, Class A | 1042 |
| Geotextile for Drainage, Type 2 | 1056 |

Construction Methods:

Rock sills shall be constructed according to the rock sill detail shown on the plans or as directed by the Engineer.  Boulders will be used for the entire sill.  One vane running perpendicular to the stream’s flow will installed so that the header rock’s top elevations are at the elevation of the proposed channel.  The sill shall run from bankfull to bankfull.  Individual rocks shall be fitted tightly such that water flows over the device and not through the individual rocks.  The header and footer boulders must be placed such that the ends butt up against each other.  No gaps between boulders will be accepted.  However, voids between the header and footer rocks can be filled with hand-placed Class A stone at the direction of the Engineer.  At the ends of the sill, a large sill rock (Structure Stone, Boulders) shall be placed in a trench immediately below bankfull elevation and backfilled with soil to meet the bankfull elevation specified on the profile.

After the rock sill is constructed, a trench shall be dug upstream along the outline of the sill. The trench shall extend a minimum of 0.5 ft below the bottom of the footer rocks.  Plate the upstream side with Geotextile, Type 2 and backfill with No 57 Stone.

Measurement and Payment

Boulders will be measured and paid for as provided in the Structure Stone Special Provision.

No. 57 Stone will be measured and paid for as provided in the Structure Stone Special Provision.

Plain Riprap, Class A will be measured and paid for as provided in the Structure Stone Special Provision.

Geotextile for Drainage Type 2 will be measured and paid for as provided in the Structure Stone Special Provision.

Such price and payment will be full compensation for all work covered by this section, including, but not limited to furnishing all materials, labor, equipment, and incidentals necessary to construct the J-hook vanes.