BOULDER ROCK AND ROLL RIFFLE :

Description

Construct and maintain boulder and stone structures placed in and along the stream at locations designated on the plans.  Boulder Rock and Roll Riffles are intended to redirect stream flow through a riffle-pool sequence and provide grade control.

The quantity of Boulder Rock and Roll Riffles to be installed will be affected by the actual conditions that occur during the construction of the project.  The quantity of Boulder Rock and Roll Riffles may be increased, decreased, or eliminated entirely as directed.  Such variations in quantity will not be considered as alterations in the details of construction or a change in the character of the work.

Materials

Refer to Division 10 of 2024 Standard Specifications for Roads and Structures

| Item | Section |
| --- | --- |
| Boulder | 1042 and SP for Structure Stone |
| No. 57 Stone | 1005 |
| Plain Riprap, Class A, B | 1042 |
| Geotextile for Drainage, Type 2 | 1056 |

Use boulders as header and footer rocks for this structure.

Construction Methods

Construct Boulder Rock and Roll Riffles in accordance with the Boulder Rock and Roll Riffle Detail shown in the plans or as directed.  Use boulders to form vane arms, footers and bank toe protection, and structural stone shall plate the riffles and pools of the structure.

Install the rock sills so that they form the angle shown in the detail when measured perpendicular to the bank toward downstream.  The top elevation of the rock sill will decrease toward the opposite bank at a slope shown in the detail, and completely extend across the channel.  The center of the rock sill shall be set at the invert elevation specified in the profile, and the vane shall extend into each bank as shown in the detail.  Place the footer boulders securely as shown in the detail, prior to placing the header boulders.  Plate the upstream side of the rock sill with Geotextile, Type 2, and backfill as specified in the detail.  Place backfill upstream of the Geotextile Type 2 plating consisting of material and dimensions shown in the plans.  Structural stone or other material shall meet the invert elevation of the riffle specified in the profile and the size gradation shall be according to the detail shown in the plans.

Excavate the downstream pool according to the profile elevations and typical pool cross-section and plate the pool with material shown in the detail or as directed by the engineer.  Place toe protection boulders along the toe of banks as indicated in the detail.

Measurement and Payment

Boulders will be measured and paid for as provided in the Structure Stone Special Provision.

No. 57 Stone will be measured and paid for as provided in the Structure Stone Special Provision.

Plain Riprap, Class A, B will be measured and paid for as provided in the Structure Stone Special Provision.

Geotextile for Drainage Type 2 will be measured and paid for as provided in the Structure Stone Special Provision.

Such price and payment will be full compensation for all work covered by this special provision, including, but not limited to furnishing all materials, labor, equipment, and incidentals necessary to construct the step pool structure.