# NCDOT M&T Form 250VM

North Carolina Department of Transportation

AGGREGATE MOISTURE REPORT

FOR VOLUMETRIC CONCRETE MOBILE MIXER

Date of   Calibration                                                      Project Number

Fine Aggregate Source

Coarse Aggregate Source

| Class of Concrete | Mix Design No. | Number of Loads | Total Yards Batched |
| --- | --- | --- | --- |

MOISTURE IN AGGREGATES

Fine Aggregate:

Trial 1                                               Time: _     _

Wet Wt. _     _     Minus             Dry Wt. _     _

- -------------------------------------------------------------------------------------------------------------------------------------------------------------------------                      =    _     _     X     100  =  _     _ % Total Moisture

Dry Wt. _     _

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………..………………..

- Total  Moisture _     _     Minus         Absorbed Moisture _     _   =    _     _ % Free Moisture

Trial 2                                               Time: _     _

Wet Wt. _     _     Minus             Dry Wt. _     _

- -------------------------------------------------------------------------------------------------------------------------------------------------------------------------                      =   _     _      X      100  =    _     _ % Total Moisture

Dry Wt. _     _

…                                                          ……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………....

- Total   Moisture _     _       Minus          Absorbed Moisture _     _  =     _     _ % Free Moisture

Coarse Aggregate:

Trial 1                                               Time: _     _

Wet Wt. _     _    Minus           Dry Wt. _     _

- -------------------------------------------------------------------------------------------------------------------------------------------------------------------------                      =   _     _          X  100    =    _     _ % Total Moisture

Dry Wt. _     _

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- Total  Moisture _     _     Minus        Absorbed Moisture _     _     =     _     _ % Free Moisture

Trial 2                                               Time: _     _

Wet Wt. _     _     Minus           Dry Wt. _     _

- -------------------------------------------------------------------------------------------------------------------------------------------------------------------------                      =   _     _          X   100    =   _     _ % Total Moisture

Dry Wt. _     _

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………..

- Total  Moisture _     _     Minus        Absorbed Moisture _     _     =     _     _ % Free Moisture

Certified Technician:                 	          Certification No.

North Carolina Department of Transportation

#### Field Calibration Report for

#### Latex Modified Concrete Mobile Continuous Mixer

Mixer Number:       	 Calibrated By:

Project: _     ____________________

Date of Calibration:       	 Mix Design:

Resident Engineer: _     _________

Step 1 - Cement Output

C.  Use a stopwatch to determine the time and meter counts required to discharge a minimum

of  100 pounds of cement into a container.

Record all weights and meter counts to the nearest tenth. Record time to the nearest hundredth of a second.

**Notes:   -        Aggregate bins must be empty during cement calibration to prevent sand and/or any stone particles from

being conveyed with the cement.

- The tolerance on cement weight is 0% to plus 4% (i.e., 100 – 104 lb). A value slightly above 104 lb may be

considered reasonably close.

D.   A minimum of 5 of these determinations (runs) shall be conducted.

|  | Run 1 | Run 2 | Run 3 | Run 4 | Run 5 |  | E. Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cement Weight in pounds |  |  |  |  |  |  |  |
| Meter Counts |  |  |  |  |  |  |  |
| Time in Seconds |  |  |  |  |  |  |  |

- Determine the Cement Output per one Meter Count:

÷        	=         	lbs.

Total Cement Weight	Total Meter Counts	Cement Output per count

- Determine the number of Meter Counts per 1 yd³ of concrete:

÷       	=

Total Design Cement Weight	Cement Output per count	Counts per 1 yd³

- Determine the Time to produce 1 yd³ of concrete:

÷        	X        	=       	sec.

Total Time	Total Cement Weight	Total Design Cement Weight	Time per 1 yd³

**Note: The calculated result “Time to produce 1 yd³ of concrete” will be used in the Fine Aggregate (Sand) Gate    Calibration,

Coarse Aggregate ( Stone) Gate Calibration, Latex Flow Calibration and Water Flow Calibration calculations.

Step 2 – Fine Aggregate (Sand) Gate Calibration

- Determine the amount of free moisture on the surface of the fine aggregate.

- Determine the weight of wet fine aggregate (sand) to be batched for 1 yd3 of concrete:

X        	=       	+       	=       	lbs.

SSD Design Sand Weight	Free Sand Moisture	SSD Design Sand Weight	Wet Sand Weight

- Determine the weight of wet fine aggregate (sand) to be batched for 1/12 yd3 of concrete:

**Note: 1/12 part of the mix will be considered optimal weight of aggregates for the calibration process only.

÷ 	12	=             	lbs.

Wet Sand Weight	Wet Sand Weight per 1/12 part of the Mix

(With tolerance ±2%:             to                lbs.)

- Determine the time to produce 1/12 yd3 of concrete:

_÷   	12	=         	sec.

Time per 1 yd³ mix                                                           Time to produce 1/12 part of the Mix

(Calculation in Step 1h)

- Adjust the gate settings to discharge the target amount of fine aggregate (sand), within ± 2%,

in the  same amount of time required to discharge 1/12 of the target cement weight per 1 yd3:

**Note: The allowable aggregate weight tolerance is ± 2%.

(The gate setting needs to be adjusted to discharge         pounds of sand ±2%, within          seconds)

|  | Run 1 | Run 2 | Run 3 | Run 4 | Run 5 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Gate Setting |  |  |  |  |  |  |
| Time in seconds |  |  |  |  |  |  |
| Sand Weight in pounds |  |  |  |  |  |  |

- It is recommended that 5 determinations (runs) of Fine Aggregate (Sand) be conducted. Additional determinations may be conducted as needed to verify consistency.

##### Fine Aggregate (Sand) Gate Setting:

Step 3 – Coarse Aggregate (Stone) Gate Calibration

- Determine the amount of free moisture on the surface of the coarse aggregate.

- Determine the weight of wet coarse aggregate (stone) to be batched for 1 yd³ of concrete:

X          	=          +        	    =               	lbs.

SSD Design Stone Weight	Free Stone Moisture	          SSD Design Stone Weight       Wet Stone Weight

- Determine the weight of wet coarse aggregate (stone) to be batched for 1/12 yd3 of concrete:

**Note: 1/12 part of the mix will be considered optimal weight of aggregates for the calibration process only.

÷	12	=                                              lbs.

Wet Stone Weight	   Wet Stone Weight per 1/12 part of the Mix

(With tolerance ±2%:       	to       	lbs.)

- Determine the time to produce 1/12 yd3 of concrete:

÷	12	=                	__sec.

Time per 1 yd³ of mix	         Time to produce 1/12 part of the Mix

(Calculation result from step 1h)

- Adjust the gate settings to discharge the target amount of coarse aggregate (stone), within ± 2%,

in the  same amount of time required to discharge 1/12 of the target cement weight per 1 yd3:

**Note: The allowable aggregate weight tolerance is ± 2%.

(The gate setting needs to be adjusted to discharge         pounds of stone ±2%, within          seconds)

|  | Run 1 | Run 2 | Run 3 | Run 4 | Run 5 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Gate Setting |  |  |  |  |  |  |
| Time in seconds |  |  |  |  |  |  |
| Stone Weight in pounds |  |  |  |  |  |  |

- It is recommended that 5 determinations (runs) of Stone be conducted.

Additional determinations may be conducted as needed to verify consistency.

##### Coarse Aggregate (Stone) Gate Setting:

Step 4 – Latex Flow Calibration

- Latex flow calibration can be determined by using the actual unit weight of latex emulsion  per gallon, or use the default value of the unit weight equals 8.5 lb./gal.

- Determine the target weight of latex emulsion per 1/6 yd3 of the concrete mix:

**Note: 1/6 part of the mix will be considered optimal weight (volume) of Latex for the calibration process only.

gal.	÷ 	6	= ____     _____________ gal.

Total Latex Volume                                        Target Latex Volume for 1/6 part of the Mix

__       gal. __________ X   	8.5_lb./gal_ =	__     __________ lbs.

Target Latex Volume for 1/6 part of the Mix 	      Unit weight of latex	Target weight of latex for 1/6 mix

(With tolerance ±3%:        to        lbs.)

- Determine the time to produce 1/6 yd3 of concrete:

____ ÷   6     =                                      sec.

Time per One c. yd. of mix                                   Target Time for 1/6 part of the Mix

(Calculation result from step 1h)

- Adjust the Latex flow setting to discharge the target weight of Latex, within ± 3%, in the same amount of time required to discharge 1/6 of the target cement weight per 1 yd3.

**Note: The allowable Latex Weight or Volume Tolerance is ±3%.

(The Latex flow setting needs to be adjusted to discharge           pounds of Latex ±3%, within         seconds)

|  | Run 1 | Run 2 | Run 3 | Run 4 | Run 5 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Latex Flow Setting |  |  |  |  |  |  |
| Time in seconds |  |  |  |  |  |  |
| Latex Weight in pounds |  |  |  |  |  |  |

- It is recommended that 5 determinations (runs) of Latex be conducted.                           Additional determinations may be conducted as needed to verify consistency.

Latex Flow Setting:

Step 5 - Water Flow Calibration

  - Determine the total weight of free water that is on the surface of the sand (FA) and  stone (CA) combined. Convert this weight to gallons by dividing by 8.33:

Sand:            	 X __     ___  _ =                           	 lbs.

SSD Design Sand Weight	Free Sand Moisture	 Weight of Free Water in Sand

Stone:    _     ________ X                       =                	 lbs.

SSD Design Stone Weight	Free Stone Moisture	   Weight of Free Water in Stone

+                 	=       	÷ _8.33_=           	_ gal.

Weight of Free Water in Sand	   Weight of Free Water in Stone    	    Volume of Free Water

  - Determine the target weight of metered water required per 1 yd3 of concrete and per 1/6 yd3of concrete:

gal.	 -            gal.	=           gal.          X _8.33_=          	lbs.

Design Water Volume	   Volume of Free Water	   Target Water Volume	           Total Water Weight

**Note: 1/6 part of the mix will be considered optimal weight (volume) of Water for the calibration process only.

lbs.	÷	6	 =                    	lbs.

Total Water Weight                                            Target Water Weight for 1/6 part of the Mix

(With tolerance ±1%:       	to        	lbs.)

  - Determine the time to produce 1/6 yd3 of concrete:

_____ ÷ 	6__ =                  	sec.

Time per 1 yd³ of mix	                      Target Time for 1/6 part of the Mix

(Calculation result from step 1h)

  - Adjust the Water flow setting to discharge the target weight of Water, within ± 1%, in the same amount of time required to discharge 1/6 of the target cement weight per 1 yd3.

**Note: The allowable Water Weight or Volume Tolerance is ±1%.

(The Water flow setting needs to be adjusted to discharge           pounds of water ±1%, within        seconds)

|  | Run 1 | Run 2 | Run 3 | Run 4 | Run 5 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Water Flow Setting |  |  |  |  |  |  |
| Time in seconds |  |  |  |  |  |  |
| Water Weight in pounds |  |  |  |  |  |  |

  - It is recommended that 5 determinations (runs) of Water be conducted.                                Additional determinations may be conducted as needed to verify consistency.

Water Flow Setting:

# Field Calibration Summary

LMC Mobile Continuous Mixer #       		__

Concrete Producer:       	__________

Date of calibration:       	_____

Project:        		_____

Calibrated by:       			 ____________________________________________________________________________________________________________________________________________________

Mix Design:       				____

Moisture during calibration: FA         		%

CA        	%

| FINE AGGREGATE GATE SETTING |  |
| --- | --- |
| COARSE AGGREGATE GATE SETTING |  |
| LATEX EMULSION FLOW SETTING |  |
| WATER FLOW SETTING |  |
| METER COUNT PER 1 CUBIC YARD OF CONCRETE |  |
| METER COUNTS PER ¼ YD³ (For Yield Test Purpose) |  |