
GUARDRAIL ANCHOR UNITS, TYPE 350 TL-2:
	(10-21-08) (Rev. 8-16-11)
	862
	SP8 R64


Description

Furnish and install guardrail anchor units in accordance with the details in the plans, the applicable requirements of Section 862 of the 2012 Standard Specifications, and at locations shown in the plans.

Materials

The Contractor may at his option, furnish any one of the guardrail anchor units or approved equal.

Guardrail anchor unit (ET-Plus) manufactured by:



Trinity Industries, Inc.



2525 N. Stemmons Freeway



Dallas, Texas  75207



Telephone:  800-644-7976

The guardrail anchor unit (SKT 350) as manufactured by:



Road Systems, Inc.



3616 Old Howard County Airport



Big Spring, Texas 79720



Telephone:  915-263-2435

Prior to installation the Contractor shall submit to the Engineer:

(A)
FHWA acceptance letter for each guardrail anchor unit certifying it meets the requirements of NCHRP Report 350, Test Level 2 in accordance with Article 106-2 of the 2012 Standard Specifications.

(B)
Certified working drawings and assembling instructions from the manufacturer for each guardrail anchor unit in accordance with Article 105-2 of the 2012 Standard Specifications.

No modifications shall be made to the guardrail anchor unit without the express written permission from the manufacturer.  Perform installation in accordance with the details in the plans, and details and assembling instructions furnished by the manufacturer.

Construction Methods

Guardrail end delineation is required on all approach and trailing end sections for both temporary and permanent installations.  Guardrail end delineation consists of yellow reflective sheeting applied to the entire end section of the guardrail in accordance with Article 1088-3 of the 2012 Standard Specifications and is incidental to the cost of the guardrail anchor unit.

Measurement and Payment

Measurement and payment will be made in accordance with Article 862-6 of the 2012 Standard Specifications.

Payment will be made under:

	Pay Item
	Pay Unit

	Guardrail Anchor Units, Type 350 TL-2
	Each


