
IMPACT ATTENUATOR UNITS, TYPE 350:
	(4-20-04) (Rev. 1-17-12)
	
	SP8 R75


Description

Furnish and install impact attenuator units and any components necessary to connect the impact attenuator units in accordance with the manufacturer’s requirement, the details in the plans and at locations shown in the plans.

Materials

The Contractor may at his option, furnish any one of the NON-GATING impact attenuator units or approved equal:

The impact attenuator unit (QUADGUARD) as manufactured by:



Energy Absorption Systems, Inc.



One East Wacker Drive



Chicago, Illinois  60601-2076



Telephone:  312-467-6750


The impact attenuator unit (TRACC) as manufactured by:



Trinity Industries, Inc.



2525 N. Stemmons Freeway



Dallas, Texas  75207



Telephone:  800-644-7976

The Contractor may at his option, furnish any one of the GATING impact attenuator units or approved equal:


The impact attenuator unit (BRAKEMASTER) as manufactured by:



Energy Absorption Systems, Inc.



One East Wacker Drive



Chicago, Illinois  60601-2076



Telephone:  312-467-6750


The impact attenuator unit (CAT) as manufactured by:



Trinity Industries, Inc.



2525 N. Stemmons Freeway



Dallas, Texas  75207



Telephone:  800-644-7976

Prior to installation the Contractor shall submit to the Engineer:

(A)
FHWA acceptance letter for each impact attenuator unit certifying it meets the requirements of NCHRP Report 350, Test Level 3, in accordance with Article 106-2 of the 2012 Standard Specifications.

(B)
Certified working drawings and assembling instructions from the manufacturer for each impact attenuator unit in accordance with Article 105-2 of the 2012 Standard Specifications.

No modifications shall be made to the impact attenuator unit without the express written permission from the manufacturer.  Perform installation in accordance with the details in the plans, and details and assembling instructions furnished by the manufacturer.

Construction Methods
If the median width is 40 feet or less, the Contractor shall supply one of the NON-GATING Impact Attenuator Units listed in the Materials Section herein.

If the median width is greater than 40 feet, the Contractor may use any of the GATING or NON-GATING Impact Attenuator Units listed in the Materials Section herein.

Measurement and Payment
Impact Attenuator Unit, Type 350 will be measured and paid at the contract unit price per each.  Such prices and payment will be full compensation for all work covered by this provision including, but not limited to, furnishing, installing and all incidentals necessary to complete the work.

Payment will be made under:

	Pay Item
	Pay Unit

	Impact Attenuator Units, Type 350
	Each


