STANDARD SPECIAL PROVISION

NCDOT GENERAL SEED SPECIFICATION FOR SEED QUALITY

	(5-17-11)
	Z-3


Seed shall be sampled and tested by the North Carolina Department of Agriculture and Consumer Services, Seed Testing Laboratory.  When said samples are collected, the vendor shall supply an independent laboratory report for each lot to be tested.  Results from seed so sampled shall be final.  Seed not meeting the specifications shall be rejected by the Department of Transportation and shall not be delivered to North Carolina Department of Transportation warehouses.  If seed has been delivered it shall be available for pickup and replacement at the supplier’s expense.

Any re-labeling required by the North Carolina Department of Agriculture and Consumer Services, Seed Testing Laboratory, that would cause the label to reflect as otherwise specified herein shall be rejected by the North Carolina Department of Transportation.

Seed shall be free from seeds of the noxious weeds Johnsongrass, Balloonvine, Jimsonweed, Witchweed, Itchgrass, Serrated Tussock, Showy Crotalaria, Smooth Crotalaria, Sicklepod, Sandbur, Wild Onion, and Wild Garlic.  Seed shall not be labeled with the above weed species on the seed analysis label.  Tolerances as applied by the Association of Official Seed Analysts will NOT be allowed for the above noxious weeds except for Wild Onion and Wild Garlic.

Tolerances established by the Association of Official Seed Analysts will generally be recognized.  However, for the purpose of figuring pure live seed, the found pure seed and found germination percentages as reported by the North Carolina Department of Agriculture and Consumer Services, Seed Testing Laboratory will be used.  Allowances, as established by the NCDOT, will be recognized for minimum pure live seed as listed on the following pages.

The specifications for restricted noxious weed seed refers to the number per pound as follows:

	Restricted Noxious
	Limitations per
	Restricted Noxious
	Limitations per

	Weed
	Lb. Of Seed
	Weed
	Lb. of Seed

	Blessed Thistle
	4 seeds
	Cornflower (Ragged Robin)
	27 seeds

	Cocklebur
	4 seeds
	Texas Panicum 
	27 seeds

	Spurred Anoda
	4 seeds
	Bracted Plantain
	54 seeds

	Velvetleaf
	4 seeds
	Buckhorn Plantain
	54 seeds

	Morning-glory
	8 seeds
	Broadleaf Dock
	54 seeds

	Corn Cockle
	10 seeds
	Curly Dock
	54 seeds

	Wild Radish
	12 seeds
	Dodder
	54 seeds

	Purple Nutsedge
	27 seeds
	Giant Foxtail
	54 seeds

	Yellow Nutsedge
	27 seeds
	Horsenettle
	54 seeds

	Canada Thistle
	27 seeds
	Quackgrass
	54 seeds

	Field Bindweed
	27 seeds
	Wild Mustard
	54 seeds

	Hedge Bindweed
	27 seeds
	
	


Seed of Pensacola Bahiagrass shall not contain more than 7% inert matter, Kentucky Bluegrass, Centipede and Fine or Hard Fescue shall not contain more than 5% inert matter whereas a maximum of 2% inert matter will be allowed on all other kinds of seed.  In addition, all seed shall not contain more than 2% other crop seed nor more than 1% total weed seed.  The germination rate as tested by the North Carolina Department of Agriculture shall not fall below 70%, which includes both dormant and hard seed.  Seed shall be labeled with not more than 7%, 5% or 2% inert matter (according to above specifications), 2% other crop seed and 1% total weed seed.

Exceptions may be made for minimum pure live seed allowances when cases of seed variety shortages are verified.  Pure live seed percentages will be applied in a verified shortage situation.  Those purchase orders of deficient seed lots will be credited with the percentage that the seed is deficient.

FURTHER SPECIFICATIONS FOR EACH SEED GROUP ARE GIVEN BELOW:

Minimum 85% pure live seed; maximum 1% total weed seed; maximum 2% total other crop seed; maximum 144 restricted noxious weed seed per pound.  Seed less than 83% pure live seed will not be approved.


Sericea Lespedeza


Oats (seeds)

Minimum 80% pure live seed; maximum 1% total weed seed; maximum 2% total other crop; maximum 144 restricted noxious weed seed per pound.  Seed less than 78% pure live seed will not be approved.


Tall Fescue (all approved varieties)

Bermudagrass


Kobe Lespedeza



Browntop Millet


Korean Lespedeza



German Millet – Strain R


Weeping Lovegrass



Clover – Red/White/Crimson


Carpetgrass

Minimum 78% pure live seed; maximum 1% total weed seed; maximum 2% total other crop seed; maximum 144 restricted noxious weed seed per pound.  Seed less than 76% pure live seed will not be approved.


Common or Sweet Sundangrass

Minimum 76% pure live seed; maximum 1% total weed seed; maximum 2% total other crop seed; maximum 144 restricted noxious weed seed per pound.  Seed less than 74% pure live seed will not be approved.


Rye (grain; all varieties)


Kentucky Bluegrass (all approved varieties)


Hard Fescue (all approved varieties)


Shrub (bicolor) Lespedeza

Minimum 70% pure live seed; maximum 1% total weed seed; maximum 2% total other crop seed; maximum 144 noxious weed seed per pound.  Seed less than 70% pure live seed will not be approved.


Centipedegrass


Japanese Millet


Crownvetch



Reed Canary Grass


Pensacola Bahiagrass


Zoysia


Creeping Red Fescue

Minimum 70% pure live seed; maximum 1% total weed seed; maximum 2% total other crop seed; maximum 5% inert matter; maximum 144 restricted noxious weed seed per pound.


Barnyard Grass


Big Bluestem


Little Bluestem


Bristly Locust


Birdsfoot Trefoil


Indiangrass


Orchardgrass


Switchgrass


Yellow Blossom Sweet Clover
