HDPE PIPE LINER:

| (1-30-14)(Rev. 1-16-24) | 300, 1044-7 | SPD 3-100 |
| --- | --- | --- |

Description

Furnish, haul and install all pipe, fittings, couplings and other material; construct joint connections; and clean out the existing pipe for the HDPE Pipe Liner inside the existing corrugated steel pipe as shown in the plans.

Materials

Refer to Division 10 of the Standard Specifications.

| Item | Section |
| --- | --- |
| High Density Polyethylene Pipe | 1044-7 |  |

Construction Methods

Install the pipe in accordance with Section 300 of the Standard Specifications.

Measurement and Payment

__-inch High Density Polyethylene Pipe Liner will be measured and paid as the actual number of linear feet of pipe liner incorporated into the completed and accepted work.  Measurement of 
__-inch High Density Polyethylene Pipe Liner will be made by counting the number of joints used and multiplying the length of the joint to obtain the number of linear feet of pipe liner installed and accepted.  Measurements of partial joints will be made along the longest length of the partial joint to the nearest 0.1 of a foot.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| __-inch High Density Polyethylene Pipe Liner | Linear Foot |