ASPHALT SURFACE TREATMENT (Cover Same Day):

| (1-30-14)(Rev. 1-16-24) | 660 | SPD 6-520 |
| --- | --- | --- |

Map(s) ______ shall receive an Asphalt Surface Treatment, Straight Seal, 78M Stone prior to receiving the Asphalt Surface Course.  The Contractor shall place only that amount of Asphalt Surface Treatment that can be covered by the Asphalt Surface Course in the same day.

All work for placing Asphalt Surface Treatment, Straight Seal, 78M Stone shall be in accordance with the requirements of Section 660 of the Standard Specifications.

The surface to be treated shall be cleaned of all dust, dirt, clay, grass, sod, and any other deleterious matter prior to the application of the asphalt surface treatment.  This material may be incorporated into the shoulder reconstruction at the direction of the Engineer.  Any excess material that cannot be incorporated into the shoulder reconstruction shall be removed and disposed of by the Contractor in an approved disposal site.

Asphalt Surface Treatment, Straight Seal, 78M Stone will be measured and paid in accordance with Article 660-11 of the Standard Specifications.