BRICK PAVERS ON CONCRETE BASE WITH BORDER:

| (1-30-14)(Rev. 1-16-24) | 825, 848 | SPD 08-350 |
| --- | --- | --- |

Description

This provision covers 4 inch concrete base and concrete border with brick pavers.  All applicable sections of the Standard Specifications Section 825 and Section 848 will apply.

Materials

Refer to Section 848 of the Standard Specifications for concrete specifications.

Brick Pavers will be __Description of type of Paver__.  Submit sample (at least 3 paver units but enough to show color variations that will be provided in the full installation) for approval prior to beginning installation.

Pavers will be first class representation of the type specified and meet all applicable standard specifications of the brick paver industry.

Construction Methods

Remove all construction and vegetative debris and add and compact subgrade material in area to be paved.  Construct expansion joints and place groove joints as directed in Article 825-10 of the Standard Specifications and as detailed in Standard Drawing No. 848.01 of the Roadway Standard Drawings.  Drill drainage holes in concrete base as detailed on the plans.  Dimensions shall be as shown on plans unless field conditions necessitate changes.  Any revisions will be at the direction of the Engineer.

Form banding with stepped concrete pavement beneath to accommodate paver depth as shown on plans and details.  Install pavers within the concrete pavement as detailed on the plans and details.  Lay pavers on full sand bed.  Joints will be tight.  Install pavers so that surface level of pavers and concrete border create a smooth continuous surface with no raised edges and no changes in level other than the intended surface slope for drainage.

Measurement and Payment

Brick Pavers on Concrete Base with Border will be measured and paid as the actual square feet of pavers, measured along the surface of the completed and accepted work.  Such price includes, but is not limited to, preparing site and subgrade, and furnishing and placing concrete, sand bed, and brick pavers.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Brick Paver on Concrete Base with Border | Square Foot |