ADJUSTMENT OF VALVE BOXES, MANHOLES, AND METER BOXES:

| (2-28-12)(Rev. 1-16-24) | 858 | SPD 8-750 |
| --- | --- | --- |

The Contractor's attention is directed to Article 858-3 of the Standard Specifications.

Adjustment to manholes, meter boxes, and valve boxes on this project shall be made by the use of an approved Rapid Set Grout, Mortar, or Concrete that will take full set and become load bearing within sixty minutes of placement.  A list of approved materials will be furnished to the Contractor by the Resident Engineer.

The Contractor shall replace worn manhole rings and covers, worn meter box frames and covers, and worn valve box frames and covers, as directed by the Engineer, with a new ring/frame and cover assembly. These assemblies will be furnished at no cost to the Contractor by the Department or utility owner.