TRIMMING AND REMOVING OF VEGETATION:

| (11-1-11) | 200-7, 1651 | SPD 16-340 |
| --- | --- | --- |

Description

Cut all undesirable weeds, vines, shrubs, and any other herbaceous and woody growth up to 8” in diameter as measured 6” from the ground.  The remaining cut vegetation shall be a maximum height of 3” for grass and weedy vegetation and a maximum height of 1” for woody vegetation.

Trim limbs along the right of way or in locations specified by the Department up to 8” in diameter.

The Department will provide traffic control.

Provide sufficient man power to clean up and remove debris from the trimming or vegetation removal operation.  For any cut woody vegetation, the Department will specify if the Contractor should remove the cut woody vegetation from the right of way and properly dispose of it or chip onsite and spread the shavings at the project site.

Equipment

Equipment requirements are site specific.  The Contractor should own or be able to lease the following equipment or equivalent:

Chipper

Mechanical Trimmer with a minimum 55’, maximum 70’ Reach

Mechanical Undergrowth Brush Cutter

Measurement and Payment

Trimming will be measured and paid as the actual number of crew hours worked.

Undergrowth Removal will be measured and paid as the actual number of crew hours worked.

Payment will include, but is not limited to, providing a four man crew, power saws, fuel, excavators, mechanical trimmer, mobilization (unless otherwise specified), trucks, chipper, equipment repairs and safety equipment necessary.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Trimming | Hour |
| Undergrowth Removal | Hour |