WOODS EDGE CLEAN-UP:

| (3-3-14)(Rev. 1-16-24) | 1650 | SPD 16-350 |
| --- | --- | --- |

Perform work in accordance with Section 1650 of the Standard Specifications unless otherwise indicated.

Clean up all woods edges as identified on the plans or as directed by the Engineer.  Remove all debris, dead, partially dead, and broken vegetation as well as undesirable underbrush, weeds, vines, and trees less than a 6” in diameter within a distance of 12 feet of the wood line edge.  In addition, limb up all trees in the indicated area to a height of 5 feet.

Perform minimal debris and undergrowth “clean-up” on existing areas that have been planted for screening or buffering purposes.  Such areas will be identified by the Engineer.

Maintain the wooded edge shall be maintained free of undesirable vegetation throughout the remaining length of the contract.

Measurement and Payment

Woods Edge Clean-Up will be measured and paid as the actual number of square yards of land measured along the surface of the ground acceptably cleaned and maintained in accordance with these specifications.

The above payment will be full compensation for all work covered by this section, including but not limited to, removal of undesirable vegetation, pruning, and debris removal.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Woods Edge Clean-Up | Square Yard |