EXISTING PLANT BED CLEAN-UP:

| (1-30-14) | 1670 | SPD 16-380 |
| --- | --- | --- |

Description

Clean up all existing plant beds as identified on the plans or as directed by the Engineer.

Construction Methods

Remove from the bed area, all undesirable grass, weeds, vines, and other woody growth.  Remove any of the existing bed plants which are damaged or in poor health.  Trim back and spray two feet around the perimeter of the bed with a total vegetation control, post-emergence herbicide.  Mulch all open areas within the beds and the entire bed area shall to a depth of 4”. A pre-emergence herbicide will be applied in the first spring following the clean-up operation.

Total vegetation control post emergence herbicide and the pre-emergence herbicide will be specified by the Engineer.

Maintain the existing plantings; free of undesirable vegetation; throughout the remaining length of the project.  Perform pruning of all existing plants according to correct and acceptable horticultural practice.

Measurement and Payment

Existing Plant Bed Clean-Up will be measured and paid as the actual number of square yards of land measured along the surface of the ground acceptably cleaned and maintained.  The above payment will be full compensation for all work covered by this section, including, but not limited to, removal of undesirable vegetation, herbicides, pruning, and mulching.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Existing Plant Bed Clean-Up | Square Yard |