HERBICIDAL TREATMENT FOR PLANT BEDS:

| (2-20-14) | 1670 | SPD 16-400 |
| --- | --- | --- |

Description

Pre-emergent total vegetation control herbicide and post-emergent herbicide shall be applied as to not harm existing plant material identified to remain by the Engineer or new plant material installed.

Materials

Use the herbicides from the following list or an approved equal:

| Herbicide Brand Name | Common Name | Formul-ation | Oral LD/50 (MG/KG) | Amount of Formulation per Acre | Lbs. of Active Ingredient per Acre | Adjuvants | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stump Control |  |  |  |  |  |  |  |
| Garlon | Triclopyr | 3 S | 2,574 | 1 gal./1 gal. of water | 3 # | 1 - 2 qts. Surfactant/acre | Paint or spray, add bullseye dye. |
| Pre-emergent |  |  |  |  |  |  |  |
| Pennant + Endurance + Gallery | Metolachlor + Prodiamine + Isoxaben | Liquid (5G) + 65 WDG + 75 DF | 3750 + >5,000 + 5,000 | 2 - 3 pts. (40#) + 2# + 1# | 1.95 - 2.93# (2#) + 20 lbs. + 1# | NA | Spring application, use tank agitation when mixing. |
| Post-emergent |  |  |  |  |  |  |  |
| Roundup | Glyphosate | 4 S | >5,000 | 2 - 4 qts. | 2 - 4 # | 2 - 4 qts. Surfactant/ 100 gals. | NA |

Construction Methods

Perform work in accordance with Article 1670-7 of the Standard Specifications and these special provisions.  Pre-emergent herbicide shall be applied as a topical application and not incorporated into the soil.  Pre-emergent as specified shall be applied if project planting has been accomplished and project is still open between March 1st and July 1st or between October 1st and December 1st.

Comply with all applicable federal, state, and local laws, ordinances, and regulations governing safety and provide all safeguards needed on his own responsibility.

Measurement and Payment

Pre-emergent Herbicidal Treatment (Plant Bed) and Post-emergent Herbicidal Treatment (Plant Bed) will be measured and paid as the actual square yards, premeasured along the surface on which herbicide is applied in accordance with these specifications

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Pre-emergent Herbicidal Treatment (Plant Bed) | Square Yard |
| Post-emergent Herbicidal Treatment (Plant Bed) | Square Yard |