PLANT BED FUMIGATION:

| (2-20-14)(Rev. 1-16-24) | 1670 | SPD 16-420 |
| --- | --- | --- |

Perform plant bed fumigation in accordance with Section 1670 of the Standard Specifications and these special provisions.  Comply with all applicable federal, state, and local laws, ordinances, and regulations governing safety and shall provide all safeguards needed on his own responsibility.  The Contractor will also be responsible for implementation of all safety precautions as listed on the Methyl Bromide label, including but not limited to, personnel safety and warning/danger signs which must be mounted on 1 inch x 1 inch wooden stakes and properly displayed.

Apply fumigant at a rate of one pound active ingredient per 100 square feet.  Inject fumigant into the soil at a depth of 4 to 6 inches and immediately covered by a 2-mil clear polyethylene fumigation cover free of holes, punctures and tears.  Perform fumigation by solid tarp rig.  Join the polyethylene strips with a red all season glue, polyolefin based adhesive to allow for easy visibility of appropriately glued sites.  The soil temperature shall be at least 55 degrees Fahrenheit at a depth of 6 inches and it shall be moderately moist (50-85% of field capacity) at the time of fumigation.