CONCRETE PAVEMENT THICKNESS VERIFICATION BY SCANNING:

| (6-20-23) |  | SPI 7-19 |
| --- | --- | --- |

Description

Sample, test, verify, and obtain acceptance of Portland Cement Concrete (PCC) pavement thickness utilizing a non-destructive magnetic imaging tomography scanning device (MIT-SCAN).  This provision applies to all PCC pavement, including mainline, shoulders, gore areas, and ramps unless otherwise stated in the Contract.

DEFINITIONS

This method utilizes a combination of MIT thickness scans, verification cores, and acceptance cores as defined below:

- Magnetic Imaging Tomography (MIT) – Scanning device used for measuring thickness of pavements in a non-destructive method.

- Quality Control Scan (QCS) – Manual scans using the MIT device where full depth coring would have traditionally been used.  2 QCS scans are required per lot.

- Scan Verification Core (SVC) – Full depth core performed at 1 QCS location every 8 lots to confirm QCS measurement verification.  SVC cores for all scans are required on the first three lots at the beginning of paving for calibration and assurance purposes.

- Quality Acceptance Core (QAC) - Full depth core performed at 1 random location every 8 lots in a different location away from the SVC to provide additional assurance of thickness conformity.

See Materials and Tests Unit for the full provision language and process

https://apps.ncdot.gov/dot/directory/authenticated/UnitPage.aspx?id=2824

Joseph D Barbour

Data Collection and Investigations Engineer

jdbarbour2@ncdot.gov

919-329-4015