## CONCRETE STEPS (with handrail):

| (Rev 11-7-08) | SPI 8-04 |
| --- | --- |

Description

Construct reinforced concrete steps with handrails in accordance with the plans and contract documents.

Materials

Refer to Division 10.

| Item | Section |
| --- | --- |
| Portland cement concrete, Class B | 1000 |
| Curing agents | 1026 |
| Steel bar reinforcement | 1070-2 |
| Galvanizing | 1076 |
| Steel Pipe Rail | ASTM A53 (schedule 40)Plain End Galvanized Pipe |

Construction Methods

Construct concrete in accordance with Section 825, except as otherwise provided herein.  Furnish and place reinforcement, as shown on the plans, in accordance with Section 425.  Give formed surfaces of the concrete a rubbed finish. Give unformed surfaces a float finish.  Compact backfill to a degree comparable to the adjacent undisturbed material.

Measurement and Payment

Concrete Steps will be measured and paid for in cubic yards computed from the dimensions shown on the plans or established by the Engineer that has been incorporated into the completed and accepted steps.  Work includes but is not limited to excavation and backfilling, furnishing and placing concrete, reinforcing steel, steel pipe rail, grout and all labor, tools, materials, equipment and incidentals necessary to complete the work.

Handrail on Steps will be measured and paid in linear feet along the top of the rail to the nearest 0.1 foot.  Such price shall include furnishing and installing handrails, grouting, painting, and all labor, tools, equipment and incidentals necessary to complete the work.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Concrete Steps | Cubic Yard |
| Handrail on Steps | Linear Foot |