## REMOVE ___'- ___" CONCRETE CURB & GUTTER:

| (11-19-24) | 1660 | SPI 8-55 |
| --- | --- | --- |

Description

Remove existing ___'-___" concrete curb & gutter in accordance with the plans, this special provision and as directed by the Engineer.

Construction Methods

Sawcut, break up, remove and satisfactorily dispose of the existing ___'-___" concrete curb & gutter to the nearest concrete joint within the limits shown in the plans or as directed by the Engineer.  Backfill with suitable soil and seed and mulch the disturbed area in accordance with Section 1660 of the Standard Specifications.

In locations where curb and gutter removal is to address ADA curb ramp compliance, sidewalks, curb ramps, and other flatwork will be paid for as found elsewhere in the contract.  Such locations will be indicated on the plans or directed by the Engineer.

Dispose of existing ___'-___" concrete curb & gutter in accordance with federal, state and local regulations will be incidental to the work.

Measurement and Payment

Remove ___'-___" Concrete Curb & Gutter will be measured and paid for in units of linear feet for the actual number of liner feet that have been completed and accepted.  Such price and payment includes, but is not limited to, providing all materials, removal and disposal of existing curb & gutter, excavating and backfilling, seeding, mulching the disturbed area, and all incidentals necessary to complete the work.  Required sidewalks, curb ramps, and other flatwork installed in lieu of backfilling with soil will be paid for as found elsewhere in the contract.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Remove ___'-___" Concrete Curb & Gutter | Linear Foot |