## REMOVE ___" CONCRETE SIDEWALK:

| (11-19-24) | 1660 | SPI 8-57 |
| --- | --- | --- |

Description

Remove existing ___" concrete sidewalk in accordance with the plans, this special provision and as directed by the Engineer.

Construction Methods

Sawcut, break up, remove, and satisfactorily dispose of the existing ___" concrete sidewalk to the nearest concrete joint within the limits shown in the plans or as directed by the Engineer.  Backfill with suitable soil and seed and mulch the disturbed area in accordance with Section 1660 in the Standard Specifications.

In locations where sidewalk removal is to address ADA curb ramp compliance, sidewalks, curb ramps, and other flatwork will be paid for as found elsewhere in the contract. Such locations will be indicated on the plans or as directed by the Engineer.

Disposal of existing ___" concrete sidewalk in accordance with federal, state and local regulations will be incidental to the work.

Measurement and Payment

Remove ___" Concrete Sidewalk will be measured and paid in units of square yards.  Such price and payment will be full compensation for removal and disposal of the existing sidewalk, backfilling, seeding, mulching the disturbed area, and all other incidentals necessary to complete the work.  Required sidewalks, curb ramps, and other flatwork installed in lieu of backfilling with soil will be paid for as found elsewhere in the contract.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Remove ___" Concrete Sidewalk | Square Yard |