## REMOVE ___" MONOLITHIC CONCRETE ISLANDS (____):

| (11-19-24) | 1660 | SPI 8-61 |
| --- | --- | --- |

Description

Remove existing ___" monolithic concrete islands (____) in accordance with the plans, this special provision and as directed by the Engineer.

Construction Methods

Sawcut, break up, remove, and satisfactorily dispose of the existing ___" monolithic concrete islands (____) within the limits shown in the plans or as directed by the Engineer.  Backfill with suitable soil and seed and mulch the disturbed area per Section 1660 in the Standard Specifications.

In locations where partial island removal is to address ADA curb ramp compliance, sidewalks, curb ramps, and other flatwork will be paid for as found elsewhere in the contract. Such locations will be indicated on the plans or directed by the Engineer.

Disposal of existing ___" monolithic concrete islands (____)  in accordance with federal, state and local regulations will be incidental to the work.

Measurement and Payment

Remove ___" Monolithic Concrete Islands (____) will be measured and paid in square yards.  Such price and payment will be full compensation for removal and disposal of the existing island, backfilling, seeding, mulching the disturbed area, and all other incidentals necessary to complete the work.  Required sidewalks, curb ramps, and other flatwork installed in lieu of backfilling with soil will be paid for as found elsewhere in the contract.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Remove ___" Monolithic Concrete Islands (____) | Square Yard |