## CONCRETE APRON FOR DROP INLET:

| (9-16-25) | 840 | SPI 8-70 |
| --- | --- | --- |

Description

Install and/or remove, dispose and replace concrete aprons at existing drainage structures in accordance with Section 840 of the Standard Specifications, the Roadway Standard Drawings No. 840.17, 840.18, 840.19, 840.26, 840.27, and 840.28, at locations indicated in the plans or as directed by the Engineer.

Materials

Use concrete apron for drop inlet materials meeting the requirements of Article 840-2 of the Standard Specifications.

Construction Methods

Remove and replace damaged concrete aprons at existing drainage structures at locations indicated in the plans or as directed by the Engineer.  Where the concrete apron does not exist, install concrete aprons at existing drainage structures at locations indicated in the plans or as directed by the Engineer.

Carefully remove frame and grate and reinstall in original conditions.  If frame and grate is damaged during operation, repairing damage at no cost to the Department.

Construct concrete apron for drop inlets in accordance with Subarticle 840-3(H) of the Standard Specifications and Roadway Standard Drawings.

Seed and mulch in accordance with Section 1660 of the Standard Specifications and seed mix special provision found elsewhere in the contract.

Dispose of materials and debris in accordance with Section 802 of the Standard Specifications.

Measurement and Payment

Concrete Apron for Drop Inlet will be measured and paid in square yards of concrete aprons that have been incorporated into the completed and accepted work.  The quantity of concrete aprons installed will be computed from the dimensions shown in the plans or from revised dimensions authorized by the Engineer.  Work shall include, but not be limited to, removal and reinstallation of the existing frame and grate construction of and placement of the new concrete apron, backfilling and compacting backfill material, installation of joint material and application of sealant, seeding and mulching all grass or soil areas disturbed by the Contractor’s operations and other incidentals necessary to complete the work.

Remove and Replace Concrete Apron for Drop Inlet will be measured and paid in square yards of removed and replaced concrete aprons that have been incorporated into the completed and accepted work.  The quantity of concrete aprons removed and replaced will be computed from the dimensions shown in the plans or from revised dimensions authorized by the Engineer.  Work shall include, but not be limited to, removal and disposal of the existing apron, removal and reinstallation of the existing frame and grate, construction of and placement of the new concrete apron, backfilling and compacting backfill material, installation of joint material and application of sealant, seeding and mulching all grass or soil areas disturbed by the Contractor’s operations and other incidentals necessary to complete the work.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Concrete Apron for Drop Inlet | Square Yard |
| Remove and Replace Concrete Apron for Drop Inlet | Square Yard |