BRIDGE CLEANING:

| (8-10-10) |  | SPD 04-200 |
| --- | --- | --- |

Description

Remove debris from bridge decks and bridge railings.

Equipment

Provide a mobile operation that includes the following: A truck mounted high pressure wash system to clean bridge decks, expansion joints, etc. with a minimum of 1,500 psi pressure. This system will include a hand held wand producing a minimum of 1,500 psi to clean weep holes and sidewalks, etc. The truck must have a minimum of eight cubic yards of debris storage capacity, provide a minimum of ten feet sweeping width, and have the capacity to store and transport a minimum of 1,800 gallons of water supply to ensure reasonable production rates.

Additionally, provide 2 truck mounted attenuators (TMA) rated for 60 mph design speed.  TMAs shall be equipped with an arrow board and flashing light.

List of Equipment

(A)	Sweeper Truck

8 C.Y. capacity minimum with arrow board and flashing light

(B)	Dump Truck/Small Tractor

Truck with arrow board and flashing light/Small tractor with front-end loader to correct water runoff from bridge deck

(C)	Truck

Truck with attenuator rated for 60 mph design speed, arrow board, and flashing light.

Construction Method

Remove all debris from bridge decks and railings.

All debris shall be deposited in a North Carolina approved sanitary landfill.  Any fees associated with the deposal of the debris is considered incidental to the bid items.

Hours of sweeping operation are 9:00 PM to 6:00 AM, Sunday through Thursday.  Contractor shall use appropriate traffic control and lighting for night time operation as stated in the 2018 Standard Specifications.

Measurement and Payment

The number of hours worked, measured to the nearest 1/4 hour, will begin when the Contractor begins work at the first job site and will continue until completion of work at the last job site.  Travel time to the first job site and from the last job site will not be paid.  Deductions shall be made for meal breaks and equipment “down” time. All equipment must be operable and all manpower present before work time can begin.

Payment will be made at the contract unit price for the various items. The above price and payment will be considered full compensation for all work covered by this provision, including but not limited to labor, hand tools, traffic control, fuel costs, and any other item(s).

All work or items necessary to complete the work other than those listed shall be considered incidental in nature and no additional compensation will be made.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Sweeper Truck | Hour |
| Dump Truck/Small Tractor | Hour |
| Truck Mounted Impact Attenuator | Hour |