TOPSOIL:

| (1-28-14) | 230-5, 1019 | SPD 05-200 |
| --- | --- | --- |

Description

Furnish and place topsoil for filling areas within proposed seeding, mulching, and planting areas, where existing vegetation has been removed or the surface disturbed, to provide proper surface drainage and to blend to surrounding grades as directed by the Engineer.

Materials

Use soil in accordance with Articles 1019-1 and 1019-2 of the 2018 Standard Specifications.  Use soil consisting of loose, friable, sandy material with a PI greater than 6 and less than 25 and a pH ranging from 5.5 to 7.0.  All topsoil shall be approved by the Engineer when delivered to the job site, whether or not the source of topsoil has been previously approved.

Construction Methods

Exercise care to minimize disturbance of roots of trees to remain and to prevent disturbance of existing water, sewer and electrical utilities which are to remain.  Place topsoil and spread evenly to the required depth which, after settlement, shall constitute finish grade.  Do not place topsoil when the ground is frozen, excessively wet, or in a condition that the soil cannot be worked easily and dressed smoothly.

Measurement and Payment

Topsoil will be measured and paid as the actual number of cubic yards of topsoil placed as specified herein and accepted.  The topsoil used on this project will be measured by pit measurement as provided in Subarticle 230-5(A) of the Standard Specifications or by truck measurement as provided by Subarticle 230-5(B) of the Standard Specifications, as directed by the Engineer.  Such price and payment will be full compensation for furnishing, placing, all labor, equipment and incidentals necessary to complete the work satisfactorily.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Topsoil | Cubic Yard |