BRICK PLANTER EDGER ON MORTAR BASE:

| (1-30-14) | 830, 848 | SPD 08-400 |
| --- | --- | --- |

Description

This provision covers brick planter edger on mortar base.  The brick planter edger is intended to create a mowing edge for interior mounded planting beds.  All applicable sections of the Standard Specifications, Section 830, Brick Masonry Construction will apply.

## Materials

Brick Pavers will be __Description of type of Paver__.  Submit sample (at least 3 paver units but enough to show color variations that will be provided in the full installation) for approval prior to beginning installation.

Pavers will be first class representation of the type specified and meet all applicable standard specifications of the brick paver industry.

All masonry materials shall meet the requirements of Section 1040 of the Standard Specifications.

### Installation

Remove all construction and vegetative debris and add and compact subgrade material in area to be paved.  Dimensions shall be as shown on plans unless field conditions necessitate changes.  Any revisions will be at the direction of the Engineer.

Install pavers on a 3 inch minimum mortar bed as shown on the plans and details.  The mortar bed shall be raised 1.5 inches towards planter interior.

Measurement and Payment

Brick Planter Edger on Mortar Base with border will be measured and paid as the actual number of linear feet, measured along the surface of the completed and accepted work.  Such price includes, but is not limited to, preparing site and subgrade, and furnishing and placing mortar, brick pavers and all incidentals.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Brick Planter Edger on Mortar Base | Linear Foot |