CLEAN DROP INLETS:

| (2-20-14) |  | SPD 8-680 |
| --- | --- | --- |

Clean trash, debris, grass, silt, and all deleterious material from all drop inlets grates as directed by the Engineer.  If required, remove the grate and clean all deleterious material from the box then replace the grate.  Obtain a new Grate from the Maintenance Department if an existing grate is broken.

Immediately place material removed from grates and boxes into trucks or onto trailers for removal.  Removed all materials from the right of way at the end of each workday.  The Contractor shall be responsible for the proper disposal of material at an approved disposal site.

Should the Contractor encounter hazardous material, operations at that location, cease work and notify the Engineer.  The Department will contact their contracted Hazardous material handler for clean-up of hazardous material.  After hazardous waste clean-up is completed, return to that location and complete the work of cleaning the affected drop inlets.

If any personnel, contractor or state, are required to enter any drainage boxes, a gas monitor shall be used.  No person shall be allowed to enter any confined space without the use of a gas monitor.

Furnish all material and labor necessary to clean drop inlets.

Measurement and Payment

Clean Drop Inlets will be measured and paid for each drop inlet cleaned to the satisfaction of the Engineer.  No direct payment will be made for costs incurred for the disposal of waste materials

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Clean Drop Inlets | Each |