RETRACING PAINT PAVEMENT LINE:

| (5-18-10) (Rev. 2-7-14) | 1205 | SPD 12-100 |
| --- | --- | --- |

Description

# Reapply plain over existing paint pavement markings on all roads included in the summary of quantities described elsewhere in the proposal.

Materials

Refer to Division 10 of the 2018 Standard Specifications.

| Item | Section |
| --- | --- |
| Paint for Pavement Marking | 1087 |  |

# Construction Method

# Broom the roadway with a mechanical broom prior to application of the paint.

# Retrace existing paint pavement markings with a single application in accordance with Section 1205 of the Standard Specifications

# Apply two applications of paint pavement marking on asphalt patched areas and resurfaced roads included in the summary of quantities described elsewhere in the proposal.

# Follow the methods outlined.

# Measurement and Payment

# Retrace Paint Pavement Lines will be measured and paid as the actual number of linear feet of pavement marking lines that have been satisfactorily placed and accepted by the Engineer per application.  The quantity of solid lines will be the summation of the linear feet of solid line measured end-to-end of the line.  The quantity of skip or broken lines will be the summation of the linear feet derived by multiplying the nominal length of a line by the number of marking lines satisfactorily placed.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Retrace Paint Pavement Line | Linear Foot |