RELENS SNOWPLOWABLE PAVEMENT MARKER:

| (3-16-10) | 1253 | SPD 12-300 |
| --- | --- | --- |

Description

Furnish and install replacement reflectors for existing snowplowable pavement markers.

Materials

Refer to Division 10 of the 2018 Standard Specifications.

| Item | Section |
| --- | --- |
| Snowplowable Pavement Markers | 1086-3 |  |

Construction Methods

Remove all existing reflectors from existing castings.  Clean the casting without removing it from the pavement.  Replace the reflector with new reflectors of the same color unless otherwise specified.  The casting shall be cleaned in accordance with the manufacturer’s recommendations prior to installation of new reflector.

Maps consisting of both replacement reflectors and new snowplowable markers shall be completed in the following order.  Replace the reflectors in all the existing marker first; followed by the new installation.

Maintain all installed reflectors prior to acceptance.

Measurement and Payment

Relens Snowplowable Pavement Marker will be measured and paid as the actual number of reflectors satisfactorily placed and accepted by the Engineer.  Payment includes, but not limited to, removing old reflectors cleaning existing castings, installing new reflector, adhesives and any other tools, equipment, labor or materials necessary to complete the work.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Relens Snowplowable Pavement Marker | Each |