LONG ARM MOWING:

| (8-22-08) | 1667 | SPD 16-620 |
| --- | --- | --- |

Description

Remove selected living trees and undesirable living undergrowth from areas of the right of way from the top of cut slopes to the bottom of the fill slopes within the limits of the equipment.

Equipment

Tractors shall comply with Section 0782 Occupational Safety Health Act Standards.  Provide tractors equipped with a minimum of two outside blinking amber lights, visible from both directions and one top of the cab mounted amber rotating light, visible from both directions.  Furnish, mount, and maintain a “Caution Mower” (W10-11) sign on the rear of each tractor.  Display company name prominently on tractor.

Provide mowers with a minimum reach of 20 feet from the centerline of the tractor to the outermost tip of the mowing blade.  Provide shields on mowers which preclude foreign objects from being thrown out from the cutting unit enclosures.  Provide a minimum of two mowers.

Provide chipper capable of processing vegetation 5 inches of less.

Construction Methods

Mow vegetation 5 inches or less hanging over the cut slope or fill slope to the limit of the mower.  Immediately remove any brush or debris from roadway.

Measurement and Payment

Long Arm Mowing will be measured and paid as the number of shoulder miles acceptably mowed.  Such price and payment will be full compensation for all work including, but not limited to, furnishing labor and equipment to mow.  No separate payment will be made for debris removal or disposal.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Long Arm Mowing | Shoulder Mile |