# CONSTRUCTION, MAINTENANCE AND REMOVAL 		(02-21-23) 
OF TeMPORARY STRUCTURE at STATION _______

## General

Construct, maintain and afterwards remove a temporary structure in accordance with the applicable parts of the Standard Specifications and this Special Provision (structure only; the approaches are not a part of this provision). Provide a temporary structure with a minimum overall length of _____ feet. Center the length of the structure about Station ________ -Detour- with the alignment, grade, and skew as indicated on the Roadway plans. If the skew is not 90°, then lengthening of the structure to accommodate a 90° skew is permitted. Provide a temporary structure with a minimum clear roadway width of _______ feet and an underclearance elevation no less than elevation __________. Temporary structures over railroads shall maintain a minimum horizontal clearance of 25’ from center of track to any temporary bent.

Design the temporary structure for HL-93 live load in accordance with the current edition of the AASHTO LRFD Bridge Design Specifications. The design of the temporary structure need not satisfy the Extreme Event I Load Combination of the AASHTO LRFD Bridge Design Specifications. Due to the expected issuance of overweight permits by the NCDOT for certain loads above legal limits, design the temporary structure to satisfy the requirements of AASHTO’s Manual for Bridge Evaluation for the following five (5) vehicle configurations:

| Truck #1 |  |  | Truck #2 |  |  | Truck #3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axle | P (k) | Distance (ft) | Axle | P (k) | Distance (ft) | Axle | P (k) | Distance (ft) |
| 1 | 12.00 | 0.00 | 1 | 12.00 | 0.00 | 1 | 4.50 | 0.00 |
| 2 | 20.00 | 8.08 | 2 | 20.00 | 8.08 | 2 | 25.00 | 8.08 |
| 3 | 20.00 | 4.00 | 3 | 20.00 | 4.00 | 3 | 25.00 | 4.00 |
| 4 | 20.00 | 4.00 | 4 | 20.00 | 4.00 | 4 | 20.00 | 18.00 |
| 5 | 16.67 | 20.00 | 5 | 18.00 | 18.00 | 5 | 20.00 | 4.00 |
| 6 | 16.67 | 4.00 | 6 | 18.00 | 4.00 |  |  |  |
| 7 | 16.66 | 4.00 |  |  |  |  |  |  |

| Truck #4 |  |  | Truck #5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Axle | P (k) | Distance (ft) | Axle | P (k) | Distance (ft) |  |  |  |
| 1 | 12.00 | 0.00 | 1 | 14.00 | 0.00 |  |  |  |
| 2 | 20.00 | 15.00 | 2 | 25.00 | 15.00 |  |  |  |
| 3 | 20.00 | 4.00 | 3 | 25.00 | 4.00 |  |  |  |
| 4 | 20.00 | 4.00 | 4 | 17.00 | 20.00 |  |  |  |
| 5 | 20.00 | 20.00 | 5 | 17.00 | 4.00 |  |  |  |
| 6 | 20.00 | 4.00 | 6 | 17.00 | 4.00 |  |  |  |
| 7 | 20.00 | 4.00 | 7 | 17.00 | 4.00 |  |  |  |

As a minimum, design the bridge rails for the AASHTO LRFD Test Level 2 (TL-2) crash test criteria, except when the plans state that a Test Level 3 (TL-3) bridge rail is required. The bridge rail design criteria are defined in the current edition of the AASHTO LRFD Bridge Design Specifications. In addition, design structural elements to which the bridge rail is attached, or elements which may receive loads transmitted through the rail, to distribute and/or withstand these loads.

Attach the bridge rails in a way that permits the bridge approach railing system to transition from the guardrail system and attach to the rigid railing system on the temporary bridge.

Submit detailed sketches of the joint assembly for review and approval. The sketches shall provide an installation procedure and dimensions depicting adequate access to install welded or bolted connections. The maximum joint opening shall be limited to three (3) inches.

Using timber floors or timber mat floors is not permitted due to anticipated high truck traffic. If timber piles are used, use piles that are new and conform to ASTM D25. Rough-peeled or clean-peeled untreated timber piles are permitted.

All wood and timber products shall be inspected in accordance with Article 1082-1 of the Standard Specifications. The use of ungraded timber and lumber is not permitted. Use material conforming to grading rules of SPIB, NELMA or other nationally recognized specification.

Submit design calculations to the Engineer for review and approval that, as a minimum, include stress calculations for the following structural components: railings, rail post, rail post connections, flooring, main girders or floor beam system, bent cap, pile bearing, pile as a structural member and longitudinal and lateral stability of pile bents if necessary. Indicate the condition of any used materials in the design calculations. Detailed drawings and design calculations of the structural components shall be signed and sealed by a North Carolina Registered Professional Engineer. For stream crossings, determine the pile stability assuming a scour depth equal to 250% of the pile diameter or width below the existing bed elevation.  The Engineer may require a more detailed analysis of scour depth for pile bents containing more than a single row of piles.

Include material specifications for all new and used materials, including commercial grades and species of timber and lumber, in the detail drawings of the structure. In addition, show the location and a detailed sketch of the used materials indicating condition of the material, the location and geometry of existing but unused holes, attachments left over from previous use and any other irregularities in the material.

New and used material for temporary structures constructed by the Contractor, including systems intended for multiple usages, shall be inspected and approved prior to assembly.

Fabricators of temporary structures utilizing modular panels shall be AISC certified or equivalent. Submit proof of certification in accordance with Section 1072 of the Standard Specifications.

All critical bolted connections in the temporary structure require new high strength bolts.  Indicate the location of the critical connections and recommended bolt size with tightening procedures in the detail drawings of the structure. The use of used high strength bolts is limited to non-critical connections and is subject to approval. For new high strength bolts, furnish the Engineer a copy of the manufacturer’s test report for each component. Have the report indicate the testing date, the location where the components were manufactured, the lot number of the material represented, the rotational capacity tests lot number and the source identification marking used by the manufacturer of each component.

Before the temporary structure is loaded, the contractor shall inspect the structure and submit a written statement certifying that the erected structure complies with the approved detailed drawings. Temporary structures utilizing modular panels shall be inspected and certified by a manufacturer’s representative. Any condition that does not comply with the accepted drawings, or any other condition deemed unsatisfactory by the Engineer, is cause for rejection.

Once vehicular traffic is allowed on a structure utilizing modular panels, routine inspection by the manufacturer will be required. The first inspection of the structure will be one (1) month after opening the structure to vehicular traffic. Subsequent inspections shall be performed every six (6) months. However, when ADTT exceeds 2,000 inspection of the temporary structure shall occur every three (3) months. An inspection report provided by the Department must be completed by the manufacturer and submitted to the Engineer within three (3) days of each inspection. Any items documented in the report indicating safety or stability issues with the structure must be reported immediately. All safety and stability repairs will be performed promptly by the Contractor and approved by the Engineer.

## Measurement and Payment

Construction, Maintenance and Removal of Temporary Structure at Sta. ____ will be paid as provided in Article 400-5 of the Standard Specifications. Such payment includes, but is not limited to, furnishing all materials, equipment, tools, labor, and incidentals necessary to complete and monitor the work.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Construction, Maintenance and Removal of Temporary Structure at Sta. ___ | Lump Sum |