EARTHEN DAM WITH SKIMMER:

Description

Provide an earthen dam with a skimmer attached to a barrel pipe at the outlet of a proposed roadway ditch to remove sediment from construction site runoff at locations shown in the erosion control plans.  See the Earthen Dam with Skimmer Detail sheet provided in the erosion control plans.  Work includes constructing earthen dam, installation of coir fiber baffles, furnishing, installation and cleanout of skimmer, providing and placing stone pad on bottom of ditch underneath skimmer device, providing and placing geotextile spillway liner, providing coir fiber mat stabilization for the skimmer outlet, removing earthen dam, coir fiber baffles, geotextile liner and skimmer device, and disposing of excess materials.

Materials

| Item | Section |
| --- | --- |
| Anchors | 1629-2 |
| Stone for Erosion Control, Class B | 1042 |
| Staples | 1060-8(D) |
| Coir Fiber Mat | 1060-14 |
| Coir Fiber Baffle | 1640 |
| Skimmer | 1644-2 |
| Low Permeability Geotextile | 1644-2 |

Construction Methods

Excavate proposed ditch according to the roadway plans and cross sections with ditch surface free of obstructions, debris, and pockets of low-density material.  Construct earthen dam and install the primary spillway according to Roadway Standard Drawings Nol 1630.09.  Construct the coir fiber baffles according to Roadway Standard Drawings No. 1640.01 and Section 1640 of the Standard Specifications.  Accumulated silt behind the earthen dam and baffles shall be removed regularly and as directed.

Install skimmer device according to manufacturer recommendations.  Install 4” Schedule 40 PVC pipe into dam on the lower side of basin 1 ft. from the bottom of the basin and according to the detail, and extend the pipe so the basin will drain.  Attach a 6 ft. arm pipe to the coupling connection and skimmer according to manufacturer recommendations.  The coupling shall be rigid and non-buoyant and not exceed a diameter of 4” and 12” in length.  Attach the rope included with the skimmer to the tee between the vent socket and the tube inlet, and the other end to a wooden stake or metal post.  Clean out skimmer device when it becomes clogged with sediment and/or debris and is unable to float at the top of water impounded in the ditch.  Take appropriate measures to avoid ice accumulation in the skimmer device.  Construct a stone pad of Class B stone directly underneath the skimmer device at bottom of ditch.  The pad shall be a minimum of 12” in height, and shall have a minimum cross sectional area of 4 ft. by 4 ft.

Line primary spillway with low permeability polypropylene geotextile unrolled in the direction of flow and lay smoothly but loosely on soil surface without creases.  Bury edges of geotextile in a trench at least 5" deep and tamp firmly.  If geotextile for the primary spillway is not one continuous piece of material, make horizontal overlaps a minimum of 18" with upstream geotextile overlapping the downstream geotextile.  Secure geotextile with eleven gauge wire staples shaped into a u shape with a length of not less than 6" and a throat not less than 1" in width.  Place staples along outer edges and throughout the geotextile a maximum of 3 ft. horizontally and vertically.  Geotextile shall be placed to the bottom and across the entire width of the ditch according to the Earthen Dam with Skimmer Detail.  Place sealant inside basin around barrel pipe on top of geotextile with a minimum width of 6 in.

At the skimmer outlet, provide a smooth soil surface free from stones, clods, or debris that will prevent contact of the coir fiber matting with the soil.  Unroll the matting and apply without stretching such that it will lie smoothly but loosely on the soil surface.  Wooden stakes, reinforcement bars, or staples may be used as anchors in accordance with the details in the plans and as directed.  Place anchors across the matting at the ends approximately 1 ft. apart.  Place anchors along the outer edges and down the center of the matting 3 ft. apart.

Measurement and Payment

The construction of the earthen dam will be paid for as Borrow Excavation as provided in Section 230 of the Standard Specifications or included in the lump sum price for grading.

Silt Excavation will be measured and paid for in accordance with Article 1630-4 of the Standard Specifications, as calculated from the typical section throughout the length of the ditch as shown on the final approved plans.

Low Permeability Geotextile will be measured and paid for in accordance with Article 1644-5 of the Standard Specifications.

Coir Fiber Baffles will be measured and paid for in accordance with Article 1640-4 of the Standard Specifications.

__" Skimmer will be measured and paid for in accordance with Article 1644-5 of the Standard Specifications.

Coir Fiber Mat will be measured and paid for in accordance with Article 1629-4 of the Standard Specifications.

Stone for Erosion Control, Class __ will be measured and paid for in accordance with Article 1610-4 of the Standard Specifications.

No measurement will be made for other items or for over excavation or stockpiling.